Server0 –

IP: 10.1.2.2

Subnet: 255.255.255.0

Gateway: 10.1.2.1

DNS: 10.1.2.2

Start IP: 10.1.2.10

Subnet: 255.255.255.0

Max user: 100

Router – Config in CLI (both interface)

en

conf t

int GigabitEthernet 0/0/1

ip address 10.1.2.1 255.255.255.0

no shut

exit

ip dhcp pool LAN

network 10.1.2.0 255.255.255.0

default-router 10.1.2.1

dns-server 10.1.2.2

exit

ip dhcp excluded-address 10.1.2.1 10.1.2.5

exit

end

write

Access point – Config

For Front of house – SSID: “IOT_FN”

For Living Room – SSID: “IOT_LR”

For Bedroom – SSID: “IOT_BR”

For Kitchen Room – SSID: “IOT_KR”

For Garage Room – SSID: “IOT_GR”

Authentication –

WPA2-PSK (Personal)

Pass Phrase: “BlackCandle” (password)

Config the Smart phone device.